| （件名） JV33 Series Resolving Main PCB Damage issue |  | 発信№ | 34QD11-006 |
| --- | --- | --- | --- |
|  |  | 発行日 | 2008/8/8 |
|  |  | 発行部門 | 品質サービス本部 |
|  |  | 承認者 | 後藤 茂 |
| 配信 | 社長、役員、事業部長、部長、MIMAKI USA INC、MIMAKI EUOPE B.V. |  |  |
| 区分 | ■指示（ □FCO ■オンコール対応 ■作業依頼 □工程改善 □その他 ） |  |  |

１．Topic

Resolving Issue of Main PCB damage

２．Contents

Currently in market, errors like unusual firing, Error07, Error200, or machine not operating, occur due to damage in main PCB.  The cause is thought to be the line cutoffs of HDCFFC Assy, so measures are taken to stop the cause of the cutoff, which is the vibrating movement of FFC.  Also, it has been established that when power OFF, the print waveform has unusual excess current, which may also be a factor to cause damage to the main PCB.  Measures are taken to control the excess current.

３．Method

Perform procedure in the next visit to user site for maintenance or repair.

※preferably ASAP.

４．Procedure

①Add sponge onto flat cable clamp on the slider PCB, then secure the 5 HDCFFC Assy with double sided tape, which will stop the FFC from moving.

②Upgrade FW to Version1.80 to stop excess current from building

→any machine without FW Version1.80 needs this upgrade.

※ HDCFFCAssy Subject Machines and Required Parts Distribution Schedule are as follows.

５．Subject Machine

Some are already fixed in factory.

- Production: Any machine in progress since 6/24 has been worked on.

- Inventory: All machines in warehouse inventory have been worked on.

- Shipped Items: Any machine shipped after 6/30 has been worked on.

- Subject Machine in market: Machines that does not fit into above①～③

| Product Name | Subject S/N |
| --- | --- |
| JV33-130 | G0704001 ～ G0805345 |
| JV33-160 | G1704001 ～ G1805329 |

６．Operation

Refer to attached Manuals.

- Manual : JV33-130/160FFC Manual / Document№D900675-13

写真：ｴﾌﾟﾄｼｰﾗと両面ﾃｰﾌﾟ

７．Required Parts

Distributed parts to secure FFC

    - Sponge Tape（image upper left）→provided for # subject machines

    - Double sided tape（image lower right）→provided for # of subject machines.

８．Distribution Amount and Date by Area

| Area | Subj # | Parts, Amount | ATTN | DATE |
| --- | --- | --- | --- | --- |
| AP | 843台 | Sponge tape, double side tape 890set | 寺島GL | 2008/8/11 |
| EU | 1501台 | Sponge tape, double side tape 1560set | ご指定ください |  |
| JP | 615台 | Sponge tape, double side tape 680set | 各拠点CS | 2008/8/11 |
| USA | 684台 | Sponge tape, double side tape 700set | Please specify |  |

- Notify of “Ship to” and ship date.

- For subject machine, see chart “JV33 subject machine”

◆HDCFFC Assy Replacement

We will replace HDCFFC Assy that have been damaged.

Please notify in advance and return replaced HDC FFCAssy

Replacement period will be until end of Sept, 2008

Contact: Quality & Service Devision Global Service G Maruyama　natsuki@mimaki.jp

Contact Quality & Service Division if you have any questions.

Thank you for your cooperation

Attachments

(1) D900675-13FFC Manual（JP/ENG）

(2) JV33 Subject Machine chart

(3) JV33 Main Unit F/W ROM File