| （Subject） JV33 Series PC Pipe BKT Wear –Off Issue Response |  | Issue № | 34QD11-005 |
| --- | --- | --- | --- |
|  |  | Date | 2008/7/30 |
|  |  | Dept | Quality Service Division |
|  |  | Approved by | Goto |
| TO | 社長、役員、事業部長、部長、MIMAKI USA INC、MIMAKI EUOPE B.V. |  |  |
|  | ■Direction（ □FCO ■OnCall Res. □Service Work □Improvement □Other ） |  |  |

Correspondence of Above Subject

１．Purpose

Market’s response action to JV33 Issue “PC Pipe BKT Wear-off”

２．Plan/Policy

Subject machines are those in operation for less than 1 year – Action taken on call.

３．Summary

Due to rough surface of PC Release Arm, PC Pipe BKT wears down causing possible problems in regular operation.

Please replace parts when half or more is worn off.

Therefore, improvements to the parts will be made.

Due to the number of reports on this wear-off issue, out of the 1320 machines manufactured between Oct 2007 and Jan 2008, 50% occurrence is estimated.

４．Parts to Replace

・Replacement of PC Pipe BKT Assy(M008120) and PC Pipe Tube Assy(M008119)

※PC Pipe Tube Assy helps prevent early depletion by the clamp touching PC pipe tube.

This part cannot be substituted, so replace at the same time with PC Pipe BKT Assy.

５．Subject Machines (Estimated)

Machines subject to this issue are not limited to those in chart below.  This indicates machines that have high possibility of occurrence

| Machine | Subject S/N（Oct ~ Jan LOT） | Defect Rate | Est. Occurrence |
| --- | --- | --- | --- |
| JV33-130 | G0710401～G0801890 | Estimate 50％ | 660 Machines |
| JV33-160 | G1710511～G1801330 |  |  |

６．Order of Action

①Please see instruction to proceed (See Guide attached to machine)

②For machines with parts to replace, there will need to be revision management.

No.5 in the Revision seal pasted on machine must be erased.

③Always replace PC Pipe BKT Assy and PC Pipe Tube Assy as a set.

７．Distribution Plan by Area

| Area | Distributed Part | Distribute Amt | ATTN | Date |
| --- | --- | --- | --- | --- |
| AP | - PC Pipe BKT Assy - PC Pipe Tube Assy | 120sets | Terashima GL | 2008/7/31 |
| EU |  | 250sets | Ikeda(Tomo)GL | 2008/7/31 |
| JP |  | 100sets | CS Each Base | 2008/7/31 |
| USA |  | 130sets | Harada | 2008/7/31 |

※Please Advise if there is change in Recipient

◆Note on Additional Distribution

If issue occurs more than estimated, we will plan to make additional shipments.  Please notify in advance.

Contact: Quality Service Division- Global Service G Maruyama　natsuki@mimaki.jp

８．Factory-Applied Machines

From below machine S/N, permanent resolution parts are already installed.

| Product | PC Pipe BKT Assy | PC Pipe Tube Assy |
| --- | --- | --- |
| JV33-130 | Applied from G0807641 |  |
| JV33-160 | Applied from G1807661 |  |

９．Related Information

・Plan Change Notice №011-3400054

・Plan Change Notice №011-3400093  / Tech.Info T-632-3400093

If you have any questions, contact Quality Service Division.

We appreciate your cooperation.

Attached Reference

(1) D900662 Replacement Guide（JP / ENG）